# [COMPANY NAME]

# Construction Safety Training Program

8-Session Comprehensive Safety Training Curriculum

This program provides eight structured training sessions covering the most critical safety topics for construction site operations. Each session includes learning objectives, detailed content, discussion points, hands-on activities, a knowledge assessment quiz, and an attendance sign-off. Sessions are designed to meet regulatory training requirements under OHSA O.Reg 213/91 (Ontario) and OSHA 29 CFR 1926 (US) while building a genuine safety culture at [COMPANY NAME].

## Program Overview

| Session | Topic | Duration |
| --- | --- | --- |
| 1 | Working at Heights & Fall Protection | 3 hours |
| 2 | Excavation & Trenching Safety | 2.5 hours |
| 3 | Scaffold Safety | 2 hours |
| 4 | Confined Space Awareness | 2.5 hours |
| 5 | Crane & Rigging Safety | 2 hours |
| 6 | Electrical Safety on Construction Sites | 2 hours |
| 7 | Fire Prevention & Hot Work | 1.5 hours |
| 8 | Environmental Compliance on Construction Sites | 1.5 hours |

Total Program Duration: 17 hours

Recommended Delivery: One session per week over 8 weeks, or compressed into a 3-day intensive boot camp.

# Pre-Training / Post-Training Assessment

Administer this assessment BEFORE Session 1 begins (pre-test) and AFTER Session 8 is complete (post-test). Comparing scores demonstrates knowledge improvement and validates the training program's effectiveness.

1. At what height does fall protection become mandatory on Ontario construction sites (O.Reg 213/91)?

Answer: _______________________________________________

2. What is the minimum anchor point strength required for a personal fall arrest system?

Answer: _______________________________________________

3. Name two methods of soil classification used to determine excavation protective systems.

Answer: _______________________________________________

4. What is the minimum duration for a post-hot-work fire watch?

Answer: _______________________________________________

5. What does LEL stand for, and what is the maximum safe reading for entry into an excavation?

Answer: _______________________________________________

6. List three components of a guardrail system as required by regulation.

Answer: _______________________________________________

7. What is the PASS technique for fire extinguisher use?

Answer: _______________________________________________

8. Who is a 'Competent Person' in the context of excavation safety?

Answer: _______________________________________________

9. What is the maximum wind speed at which most elevated work must be suspended?

Answer: _______________________________________________

10. Name two environmental compliance measures required on construction sites to manage stormwater.

Answer: _______________________________________________

Pre-Test Score: _____ / 10    |    Date: ____________    |    Evaluator: ____________

Post-Test Score: _____ / 10    |    Date: ____________    |    Evaluator: ____________

Improvement: _____ points

# Session 1: Working at Heights & Fall Protection

Duration: 3 hours  |  Delivery: Classroom (1.5 hours) + Hands-On (1.5 hours)

## Learning Objectives

Identify when fall protection is legally required under Ontario (O.Reg 213/91 s.26) and OSHA (29 CFR 1926 Subpart M) regulations

Explain the hierarchy of fall protection and select the appropriate method for each work situation

Conduct a 10-point pre-use harness inspection and properly don a full-body harness

Calculate fall clearance distance to verify adequate clearance below the work surface

Develop and communicate a rescue plan before beginning work at height

## Content

A. Legal Requirements

Ontario O.Reg 213/91 Section 26: Fall protection required at 3 meters (10 feet) on construction projects. Working at Heights (WAH) training required — approved by Chief Prevention Officer, valid 3 years.

OSHA 29 CFR 1926.501: Fall protection required at 6 feet (1.8 meters) in construction. Each employee on a walking/working surface with an unprotected side or edge 6 feet or more above a lower level must be protected.

[COMPANY NAME] Policy: Where regulatory standards differ (e.g., multi-jurisdictional projects), apply the MORE RESTRICTIVE standard.

Employer obligations: provide fall protection systems, training, equipment inspection, and rescue capability. Worker obligations: use equipment as trained, report defects, refuse unsafe work.

B. Hierarchy of Fall Protection

Level 1 — Elimination:

Can the task be completed from ground level? Prefabrication, tilt-up construction, ground-level assembly, and mechanical lifting can eliminate the need for workers at height entirely. ALWAYS consider elimination first.

Level 2 — Guardrails:

Passive protection — does not require worker action. Top rail at 42 inches (1070mm), mid-rail at 21 inches, toe board minimum 3.5 inches. Posts at maximum 8-foot spacing. Must withstand 200 lbs applied force at any point on the top rail.

Level 3 — Safety Nets:

Positioned below the working surface (maximum 30 feet below). Specialized installation by competent person. Used primarily in bridge, structural steel, and high-rise construction.

Level 4 — Personal Fall Arrest Systems (PFAS):

Active protection — requires worker training and proper use. Components: full-body harness + connecting device (lanyard or SRL) + anchor point. Last resort in the hierarchy — use only when higher-level controls are not feasible.

C. Harness Inspection: 10-Point Pre-Use Check

Webbing (all straps): Check for cuts, fraying, abrasion, chemical burns, UV damage, heat damage, or discoloration

Load-bearing stitching: Inspect for broken, pulled, or cut threads on all connection points

Dorsal D-ring: Check for cracks, corrosion, deformation, sharp edges — must pivot freely

Sternal D-ring (if equipped): Same inspection as dorsal

Buckles and adjusters: Verify proper latching, smooth adjustment, and release function — no bent or corroded parts

Leg straps: Inspect webbing and buckles — these bear significant load during fall arrest

Chest strap: Verify buckle function and proper webbing condition

Labels and markings: Confirm manufacturer label is legible — model, serial number, date of manufacture, standards compliance (CSA Z259.10 / ANSI Z359.11)

Previous fall arrest: If any component shows signs of deployment (stretched shock absorber, deformed D-ring, elongated webbing) — DESTROY the equipment. It cannot be re-used.

Overall condition: Confirm equipment is within manufacturer's service life (typically 5 years from first use, 10 years from manufacture — check manufacturer guidelines)

D. Lanyard Types and Selection

Shock-absorbing lanyard: 6-foot maximum length with integrated energy absorber (decelerator pack). Limits maximum arrest force on the body to 8 kN (1,800 lbs). Requires 18.5 feet minimum fall clearance.

Self-retracting device (SRD/SRL): Allows greater range of movement. Cable or webbing retracts into housing like a seatbelt. Locks instantly in a fall — limits free-fall to inches. Preferred for leading-edge work, steel erection, and confined vertical movement.

Positioning lanyard: Adjustable lanyard used to position a worker (e.g., working on a column, tying rebar). Does NOT arrest falls. Must always be used WITH a separate fall arrest system.

Horizontal lifeline: Engineered cable or webbing system allowing multiple workers to move laterally along an edge while remaining connected. Must be designed by a professional engineer.

E. Anchor Point Requirements

Minimum strength: 5,000 lbs (22.2 kN) per attached worker — or 2x the maximum arrest force if certified by a professional engineer

Anchor above the D-ring when possible — reduces free-fall distance

NEVER anchor to: conduit, piping, ductwork, non-rated structural members, guardrails, or scaffold frames

Acceptable anchors: structural steel beams (with beam clamp), concrete with through-bolted anchor, engineered roof anchors, horizontal lifeline systems

F. Fall Distance Calculation

Total clearance needed = Free-fall distance + Deceleration distance + Harness stretch + Worker height (D-ring to feet) + Safety margin

Standard calculation: 6 ft (lanyard) + 3.5 ft (deceleration) + 1 ft (harness) + 6 ft (worker) + 2 ft (safety) = 18.5 feet minimum clearance below the anchor point

If clearance is less than 18.5 feet, use an SRL (reduces free-fall to inches) or a shorter lanyard

G. Rescue Planning

A rescue plan MUST be in place BEFORE any worker goes to height — it is not acceptable to 'figure it out if something happens'

Suspension trauma: blood pools in the legs when a worker is suspended motionless in a harness. Loss of consciousness can occur within 5–15 minutes. Death within 15–30 minutes.

Rescue must be achievable within 15 minutes of a fall arrest event

Rescue options: self-rescue (trauma strap — worker stands in a loop to relieve leg pressure), peer rescue (trained co-worker lowers the fallen worker using a rescue kit), aerial lift rescue (boom lift or scissor lift positioned nearby), emergency services (CALL 911 — but do not rely on this as the primary plan)

Communicate the rescue plan to ALL workers at height before they ascend — everyone knows their role

H. Ladder Safety

Position at 1:4 ratio (1 foot out for every 4 feet of height) — the shoe test: stand at the base, extend arms straight — palms should rest on the rung at shoulder height

Extend 3 feet (1 meter) above the landing

Secure at the top (tie-off) — never rely on friction alone

3-point contact at all times: two hands and one foot, or two feet and one hand

Face the ladder when climbing or descending — never climb with your back to the ladder

Never carry tools while climbing — use a tool belt, tool bucket, or tag line

Inspect before each use: check rungs, rails, feet, locks, labels. Damaged ladders are removed from service immediately.

I. Scaffold Safety Overview

Erection, modification, and dismantling by competent person only

Daily inspection before use: foundations, frames, braces, planks, guardrails, ties, access

Tag system: GREEN = safe, YELLOW = restricted access (competent person only), RED = do NOT use

Never exceed rated load capacity (light duty 25 psf, medium 50 psf, heavy 75 psf)

Access via internal ladder or stair tower — NEVER climb the scaffold frame

J. Hands-On Activity (1.5 hours)

Practice: 10-point harness inspection — each participant inspects a harness and identifies intentional defects planted by the instructor

Practice: Harness donning — each participant dons a harness, adjusts straps, and is checked by the instructor for proper fit

Practice: Anchor point identification — walk through the training area or job site and identify acceptable and unacceptable anchor points

Practice: Fall clearance calculation — participants calculate clearance for three different scenarios

Discussion: Develop a rescue plan for a specific scenario (worker falls from a scaffold, worker falls from a leading edge, worker falls from a ladder)

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. At what height is fall protection required on Ontario construction sites?

Answer: _______________________________________________

Q2. List the four levels of the fall protection hierarchy in order of preference.

Answer: _______________________________________________

Q3. What is the minimum anchor point strength for a personal fall arrest system?

Answer: _______________________________________________

Q4. Name three things you check during a harness pre-use inspection.

Answer: _______________________________________________

Q5. What is the total fall clearance distance needed when using a 6-foot shock-absorbing lanyard?

Answer: _______________________________________________

Q6. Why must a rescue plan be in place BEFORE work at height begins?

Answer: _______________________________________________

Q7. What are the three components of a personal fall arrest system (PFAS)?

Answer: _______________________________________________

Q8. What is suspension trauma and how quickly can it become life-threatening?

Answer: _______________________________________________

Q9. What is the proper angle for setting a portable ladder?

Answer: _______________________________________________

Q10. Can a harness that has arrested a fall be re-used? Explain your answer.

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 1: Working at Heights & Fall Protection ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 2: Excavation & Trenching Safety

Duration: 2.5 hours  |  Delivery: Classroom

## Learning Objectives

Define the role and responsibilities of a Competent Person for excavation safety

Classify soil types (A, B, C) using visual and manual testing methods

Select appropriate protective systems based on soil classification

Identify requirements for utility locates, access/egress, and atmospheric monitoring

Describe emergency rescue procedures for excavation incidents

## Content

A. Competent Person Requirements

Definition: A person who is capable of identifying existing and predictable hazards, has authority to take prompt corrective measures, and has training/experience in soil analysis and protective systems

Must be designated by the employer for EACH excavation

Authority to remove workers and stop work without waiting for management approval

Responsible for daily inspections, soil classification, and protective system selection

B. Soil Classification

Type A: Most stable — clay, silty clay, hardpan. Unconfined compressive strength > 1.5 tsf. Cemented soils. No fissures, no seepage, not vibration-affected.

Type B: Moderate stability — silt, sandy loam, medium clay, angular gravel, previously disturbed Type A. Compressive strength 0.5 to 1.5 tsf.

Type C: Least stable — gravel, sand, loamy sand, submerged soil, soil with freely seeping water. Compressive strength < 0.5 tsf. Previously disturbed soils.

Visual tests: soil type, crack patterns, water seepage, layering, evidence of previous disturbance, vibration exposure

Manual tests: pocket penetrometer (compressive strength reading), thumb penetration (Type C if thumb penetrates easily), ribbon test (clay ribbons = cohesive = more stable), dry strength test

If layered: classify based on the WEAKEST layer in the excavation wall

C. Protective Systems by Soil Type

Type A: Maximum slope 3/4:1 (53 degrees). Benching permitted (4-foot maximum bench height).

Type B: Maximum slope 1:1 (45 degrees). Benching permitted (4-foot maximum bench height).

Type C: Maximum slope 1-1/2:1 (34 degrees). Benching NOT permitted.

Alternative: Shoring (hydraulic aluminum, timber, pneumatic) — supports the excavation walls mechanically

Alternative: Shielding (trench box) — protects workers inside the shield from wall collapse. Workers must remain inside the shield at all times.

Engineered systems: any system not covered by tabulated data must be designed by a professional engineer

D. Utility Locates (Call Before You Dig)

Ontario: ON1Call — call or submit online minimum 5 business days before excavation

US: 811 — national call-before-you-dig number, minimum 2–3 business days (varies by state)

Locates identify: gas, electrical, water, sewer/storm, telecom, cable, fiber

Hand dig (pothole) within 1 meter of any located utility before mechanical excavation

If an unmarked utility is encountered: STOP WORK, notify Superintendent, verify with utility owner

Utility strike protocol: STOP immediately. Gas — evacuate, no ignition sources, ventilate. Electrical — assume energized, clear area. Water — control flow, minimize erosion. Call 911 + utility owner.

E. Entry/Exit Requirements

Ladders within 25 feet of lateral travel from any worker in excavations > 4 feet deep

Ladders extend 3 feet above top of excavation, secured against displacement

Ramps must be non-slip surface with cleats if used for worker access

Structural ramps for vehicles must be designed by competent person

F. Water Accumulation Management

Dewater before entry — pump must run continuously during occupancy

Divert surface water with berms and ditches

Re-classify soil after significant water exposure (water can change Type A to Type C)

If water is flowing or accumulating faster than pumps can remove it, evacuate immediately

G. Atmospheric Hazards

Sources: decomposing organic material (H2S, methane), proximity to gas lines, contaminated soil, landfill proximity

Testing: calibrated 4-gas monitor — O2 (19.5%–23.5%), LEL (< 10%), H2S (< 10 ppm), CO (< 25 ppm)

Test before entry, at the bottom, and continuously if hazard potential exists

If readings are outside safe limits: evacuate, ventilate, re-test. If not correctable, treat as permit-required confined space.

H. Heavy Equipment Setback

Equipment vibration can cause excavation wall failure

Minimum setback based on soil type and equipment weight (Type C requires double Type A distances)

Use stop blocks, berms, or physical barriers to prevent equipment from approaching the edge

Operator and ground crew communication via radio or hand signals at all times

I. Emergency Rescue

A rescue plan must be in place BEFORE workers enter any excavation over 4 feet

Do NOT enter a collapsed excavation to rescue a trapped worker — secondary collapse risk is extremely high

Call 911 immediately. Secure the perimeter. Prevent additional workers from entering.

Provide information to emergency responders: depth, soil type, location of trapped worker(s), utilities in the area

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. What is the definition of a 'Competent Person' for excavation safety?

Answer: _______________________________________________

Q2. Name the three soil types and give the maximum allowable slope angle for each.

Answer: _______________________________________________

Q3. What manual test can you perform to determine if soil is Type C?

Answer: _______________________________________________

Q4. How far from the excavation edge must spoil piles be placed (minimum)?

Answer: _______________________________________________

Q5. At what excavation depth do access/egress requirements apply (ladders within 25 feet)?

Answer: _______________________________________________

Q6. What are the four gases monitored by a standard 4-gas detector in excavation work?

Answer: _______________________________________________

Q7. Why is benching NOT permitted in Type C soil?

Answer: _______________________________________________

Q8. What is the minimum number of business days required for a utility locate request in Ontario?

Answer: _______________________________________________

Q9. What should you do if you strike a gas line during excavation?

Answer: _______________________________________________

Q10. Why should you NEVER enter a collapsed trench to rescue a trapped worker?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 2: Excavation & Trenching Safety ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 3: Scaffold Safety

Duration: 2 hours  |  Delivery: Classroom

## Learning Objectives

Identify common scaffold types used on construction sites and their applications

Describe the requirements for scaffold erection, daily inspection, and dismantling

Explain load capacity ratings and falling object protection requirements

Recognize weather-related hazards that affect scaffold safety

## Content

A. Scaffold Types

Frame scaffold: Most common — modular steel frames, cross braces, planking, and guardrails. Used for masonry, cladding, painting, and general access.

System scaffold (ring-lock, cup-lock): Modular components with rosette/cup connections. Faster erection, stronger connections. Used for complex geometries.

Suspended scaffold (swing stage): Platform suspended from the roof by wire ropes. Used for window installation, curtain wall, exterior finishing on high-rise buildings.

Mast climbing work platform: Motorized platform that climbs a mast attached to the building. Used for high-rise exterior work requiring heavy loads.

B. Erection Requirements

Erection, modification, and dismantling by or under the direct supervision of a Competent Person ONLY

Foundation: level ground, mudsills or base plates, screw jacks to level (never use concrete blocks or unstable supports)

Plumb and level: frames must be plumb and level before planking — check with a level at each lift

Bracing: all cross braces and horizontal braces installed per manufacturer instructions

Ties to the building: at intervals specified by manufacturer (typically every 26 feet vertically, 30 feet horizontally)

Planking: scaffold-grade lumber or manufactured planks, fully spanning between supports, no gaps > 1 inch, extending minimum 6 inches beyond supports (maximum 12 inches overhang)

C. Daily Inspection

Inspect EVERY day before use: foundations, frames, braces, planks, guardrails, ties, access points

Tag system: GREEN tag = inspected and safe. YELLOW tag = restricted access — competent person authorization required. RED tag = DO NOT USE — unsafe condition.

After windstorm, heavy rain, freezing rain, or any event that could affect scaffold integrity — re-inspect before use

Check for: settling of foundations, loose or missing braces, cracked welds, missing guardrails, damaged planking, debris on platforms

D. Load Capacity

Light duty: 25 psf — one worker with light tools (painting, caulking)

Medium duty: 50 psf — two workers with tools and some materials (masonry, cladding)

Heavy duty: 75 psf — three or more workers with heavy materials (bricklaying with full mortar tubs)

NEVER exceed the rated capacity — know the rating for your scaffold and manage material loading accordingly

Material should be evenly distributed on the platform — avoid point loading

E. Access

Access via internal ladder, stair tower, or designated access point ONLY

NEVER climb the scaffold frame (cross braces, end frames) — this is a leading cause of scaffold-related falls

Ladder access: through trap doors in platforms, with proper guardrail protection at each level

F. Falling Object Protection

Toe boards: minimum 3.5 inches high at all open edges to prevent tools and materials from falling

Screening/debris netting: required when scaffold is adjacent to public areas or occupied lower levels

Barricade the ground area below active scaffold work — warning signs, hard hat required zone

Workers below active scaffold work must wear hard hats and maintain awareness

G. Weather Conditions

Wind: suspend scaffold work when sustained wind speeds exceed 40 km/h (25 mph). For suspended scaffolds, typical limit is 35 km/h (22 mph).

Ice/snow: clear platforms of ice and snow before use. Apply salt or sand if needed. If platforms cannot be cleared safely, suspend scaffold work.

Lightning: evacuate all scaffolds immediately at the first sign of lightning — scaffolds are conductive and elevated.

H. Dismantling

Reverse order of erection — top down

Under competent person supervision at all times

Remove planking, then guardrails, then braces, then frames — one lift at a time

Lower components safely — never throw scaffold parts from height

Workers dismantling must wear fall protection (harness) when at height without guardrails

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. Name three types of scaffolding commonly used on construction sites.

Answer: _______________________________________________

Q2. Who may erect, modify, or dismantle scaffolding?

Answer: _______________________________________________

Q3. What does a RED scaffold tag mean?

Answer: _______________________________________________

Q4. What is the load capacity rating (in psf) for medium-duty scaffolding?

Answer: _______________________________________________

Q5. How should workers access scaffold platforms — and what method should NEVER be used?

Answer: _______________________________________________

Q6. What is the minimum toe board height required to prevent falling objects?

Answer: _______________________________________________

Q7. At what wind speed must most scaffold work be suspended?

Answer: _______________________________________________

Q8. How often must scaffolding be inspected?

Answer: _______________________________________________

Q9. In what order should scaffolding be dismantled?

Answer: _______________________________________________

Q10. What fall protection is required for workers dismantling scaffold at height?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 3: Scaffold Safety ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 4: Confined Space Awareness

Duration: 2.5 hours  |  Delivery: Classroom

## Learning Objectives

Define a confined space and distinguish between permit-required and non-permit confined spaces

Identify atmospheric hazards and explain gas monitor operation and alarm thresholds

Describe the entry permit procedure including attendant, entrant, and entry supervisor roles

Explain rescue planning requirements and the critical rule that the attendant must NEVER enter to rescue

## Content

A. Definition

A confined space has three characteristics: (1) enclosed or partially enclosed, (2) not designed or intended for continuous human occupancy, (3) has or may develop a hazardous atmosphere, or has the potential for engulfment, entrapment, or other recognized hazard

Construction examples: manholes, vaults, tanks, silos, tunnels, caissons, deep excavations, utility pits, pipe runs, boiler interiors, ductwork

B. Permit-Required vs. Non-Permit

Permit-required: contains or has potential for a hazardous atmosphere, engulfment material, converging walls/floors, or other serious recognized hazards

Non-permit: meets the confined space definition but does not have the above hazards. Can be reclassified to permit-required if conditions change.

When in doubt — treat as permit-required. The permit process protects lives.

C. Atmospheric Testing

Test BEFORE entry — never enter to test. Lower the gas monitor or use a remote sampling pump.

4-gas monitor (minimum): O2 (19.5%–23.5%), LEL (< 10% — alarm typically at 10%), H2S (< 10 ppm), CO (< 25 ppm)

Test at multiple levels: gases stratify — heavier gases (H2S) sink, lighter gases (methane) rise, oxygen depletion occurs at various levels

Continuous monitoring during occupancy — conditions can change rapidly

Calibrate gas monitors per manufacturer schedule (typically daily bump test, monthly full calibration)

D. Ventilation

Natural ventilation: may be sufficient for large, well-ventilated spaces with confirmed safe atmosphere

Mechanical ventilation: forced air blower with flexible duct, directing fresh air to the bottom of the space (displacement ventilation)

NEVER ventilate with pure oxygen — creates an oxygen-enriched atmosphere (fire/explosion risk)

Ventilate before entry and continuously during occupancy

E. Entry Permit Procedure

Entry Supervisor (Superintendent or Safety Officer): authorizes the entry, verifies all precautions, signs the permit

Attendant: stationed at the entry point at all times. Maintains communication with entrants. Monitors conditions. Summons rescue if needed. NEVER ENTERS THE SPACE TO RESCUE — this is the most critical rule in confined space safety.

Entrant: trained in confined space hazards, monitoring, and emergency procedures. Wears a harness with retrieval line (when feasible). Exits immediately when ordered by attendant or when alarm sounds.

Permit duration: typically one shift (maximum 12 hours). New permit required for each shift or if conditions change.

F. Lockout/Tagout of Connected Systems

Isolate all energy sources that feed into or through the confined space: electrical, mechanical, hydraulic, pneumatic, thermal, chemical

Lock out and tag out each energy source per [COMPANY NAME]'s LOTO procedure

Blank or disconnect all piping that could introduce liquids, gases, or materials into the space

Verify zero energy — test after lockout to confirm isolation is effective

G. Rescue Planning

A rescue plan must be in place BEFORE any entry — this is non-negotiable

Non-entry rescue (preferred): mechanical retrieval system (tripod with winch) extracts the entrant without anyone entering the space

Entry rescue: trained rescue team enters the space with self-contained breathing apparatus (SCBA). Only when non-entry rescue is not feasible.

Response time: rescue must be achievable within minutes — delays in confined space rescues are almost always fatal

The attendant NEVER enters to rescue. More than 60% of confined space fatalities are would-be rescuers.

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. What three characteristics define a confined space?

Answer: _______________________________________________

Q2. What is the difference between a permit-required and non-permit confined space?

Answer: _______________________________________________

Q3. What are the four gases measured by a standard 4-gas monitor and what are their safe limits?

Answer: _______________________________________________

Q4. At what levels in a confined space should atmospheric testing be performed?

Answer: _______________________________________________

Q5. What is the single most important rule for a confined space attendant?

Answer: _______________________________________________

Q6. Why should you NEVER ventilate a confined space with pure oxygen?

Answer: _______________________________________________

Q7. What is lockout/tagout and why is it critical for confined space entry?

Answer: _______________________________________________

Q8. What type of rescue is preferred for confined space emergencies?

Answer: _______________________________________________

Q9. How often must gas monitors be bump-tested?

Answer: _______________________________________________

Q10. More than 60% of confined space fatalities involve what group of people?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 4: Confined Space Awareness ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 5: Crane & Rigging Safety

Duration: 2 hours  |  Delivery: Classroom

## Learning Objectives

Identify common crane types on construction sites and basic operational principles

Describe signal person requirements and standard hand signals

Explain load chart concepts and the importance of never exceeding rated capacity

Inspect rigging hardware and understand sling angle load reduction

Identify powerline proximity hazards and minimum safe distances

## Content

A. Crane Types on Construction Sites

Mobile crane (hydraulic truck crane, crawler crane): travels on wheels or tracks. Variable capacity based on configuration (boom length, radius, counterweight).

Tower crane: fixed to a concrete foundation or building. High reach, moderate capacity. Used for high-rise and large commercial projects.

Boom truck (knuckle boom, stiff boom): truck-mounted crane for material delivery and placement. Lower capacity than mobile cranes.

B. Signal Person Requirements

A signal person is required when the crane operator cannot see the load, the landing zone, or the travel path

Must be a designated, trained individual — not just anyone standing nearby

Wears a high-visibility vest or gloves distinct from other workers

Uses standard hand signals (CSA/ASME B30.5) or radio communication

Standard signals: Hoist (arm extended, index finger pointing up, rotate finger in small circles), Lower (arm extended, index finger pointing down, rotate), Stop (arm extended, palm down — DO NOT MOVE), Emergency Stop (both arms extended, palms down — ALL STOP)

C. Load Charts and Capacity

Every crane has a load chart specific to its configuration — posted in the operator cab

Load chart shows maximum lift capacity based on: boom length, operating radius (distance from center of rotation to load), boom angle, counterweight, and ground conditions

NEVER exceed rated capacity — crane tip-over and structural failure are catastrophic and often fatal

The operator must know the weight of every load BEFORE lifting — weigh tickets, engineering calculations, or rigging tables

Wind, dynamic loading (swinging), and ground conditions reduce effective capacity

D. Lift Planning

Every lift must be planned: weight of load + rigging, rigging configuration, crane setup position, radius, ground conditions, obstacles, powerlines, wind

Critical lifts (typically > 75–80% of crane capacity, or lifts over occupied areas) require a written lift plan reviewed by a professional engineer

Ground conditions: verify soil bearing capacity. Use crane mats, outrigger pads, or engineered supports. Soft ground = reduced capacity or tip-over risk.

E. Rigging Hardware Inspection

Wire rope slings: inspect for broken wires (remove if 10 or more randomly distributed in one lay length), kinks, crushing, bird-caging, corrosion, heat damage, end fitting damage

Synthetic web slings: inspect for cuts, tears, snags, abrasion, chemical burns, knots, UV degradation, melted fibers, damaged fittings

Chain slings: inspect for stretched links, cracks, gouges, wear (exceeding manufacturer limits), twisted or bent links

Shackles: check for proper pin engagement, cotter pin in place, no visible deformation or cracks. Stamped capacity must be legible.

Hooks: safety latch functional, no throat opening beyond manufacturer limit, no twist or crack at the saddle

F. Sling Angles and Load Reduction

As sling angle decreases from vertical, the load on each sling leg INCREASES dramatically

90 degrees (vertical): 100% of sling rated capacity available

60 degrees: 87% of rated capacity available

45 degrees: 71% of rated capacity available

30 degrees: 50% of rated capacity available — MINIMUM acceptable sling angle

Below 30 degrees: sling failure risk — do NOT rig below 30 degrees without engineering approval

G. Powerline Proximity

Minimum approach distances (crane, load, rigging, tag lines — ALL parts):

Up to 750V: 3 meters (10 feet)

750V to 150kV: 5 meters (15 feet)

150kV to 250kV: 6 meters (20 feet)

Above 250kV: 8 meters (25 feet) — or as specified by utility owner

If work must occur within these distances: contact the utility owner, de-energize the line, install physical barriers, and use a dedicated powerline spotter

NEVER assume a powerline is dead — treat all powerlines as energized

H. Tag Lines and Load Control

Tag lines are used to control load swing and rotation during lifting and landing

Use non-conductive tag lines (polypropylene) when working near electrical hazards

Workers handling tag lines must never wrap the line around their hand, arm, or body — if the load shifts, you must be able to release instantly

NEVER walk under a suspended load — no exceptions

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. Name two types of cranes commonly found on construction sites.

Answer: _______________________________________________

Q2. When is a signal person required for crane operations?

Answer: _______________________________________________

Q3. What does a load chart tell the crane operator?

Answer: _______________________________________________

Q4. What is the minimum acceptable sling angle, and what percentage of rated capacity is available at that angle?

Answer: _______________________________________________

Q5. What is the minimum approach distance to a powerline carrying up to 750 volts?

Answer: _______________________________________________

Q6. Name three things to inspect on a wire rope sling before use.

Answer: _______________________________________________

Q7. What defines a 'critical lift' and what additional planning does it require?

Answer: _______________________________________________

Q8. Why must you never wrap a tag line around your hand?

Answer: _______________________________________________

Q9. What should you assume about every powerline on or near a construction site?

Answer: _______________________________________________

Q10. What is the hand signal for 'Emergency Stop' during crane operations?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 5: Crane & Rigging Safety ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 6: Electrical Safety on Construction Sites

Duration: 2 hours  |  Delivery: Classroom

## Learning Objectives

Explain GFCI requirements and daily testing procedures for temporary power on construction sites

Describe lockout/tagout procedures for electrical work on construction projects

Identify overhead powerline hazards and minimum safe approach distances

Recognize wet condition electrical hazards and emergency response for electrical shock

## Content

A. GFCIs (Ground Fault Circuit Interrupters)

Required on ALL temporary power outlets on construction sites (OHSA/OSHA requirement)

GFCIs detect ground faults (current leaking to ground through a person or equipment) and disconnect power in 1/40th of a second

Test GFCIs DAILY before use: press TEST button (should trip), press RESET button (should restore power). If it does not trip, remove from service.

GFCI protection applies to all 120V, 15A and 20A circuits on construction sites — no exceptions

Portable GFCI adapters can be used when panel-mounted GFCIs are not available

B. Lockout/Tagout (LOTO)

Required before any electrical work begins — maintenance, troubleshooting, or installation of new circuits

Procedure: identify all energy sources, notify affected workers, shut off power, lock out at the disconnect/breaker with a personal padlock, tag with worker name and date, verify zero energy (test with a voltage tester)

One lock per worker — each worker controls their own lock. No one removes another person's lock.

Try-out: after lockout, attempt to start the equipment or energize the circuit to confirm isolation is effective

Removal: only the person who installed the lock may remove it. If worker is absent, follow [COMPANY NAME]'s lock removal procedure (management authorization, verification of safe conditions).

C. Overhead Powerline Safety

Leading cause of electrocution on construction sites — contact by cranes, aerial lifts, metal ladders, rebar, and scaffolding

Minimum approach distances apply to ALL conductive objects (cranes, scaffolds, equipment, materials):

Up to 750V: 3 meters (10 feet). 750V–150kV: 5 meters (15 feet). Above 150kV: per utility owner requirements.

Identify overhead powerlines during pre-project planning — mark locations, post warning signs, install physical barriers (flag lines, goalposts)

If work must be performed within minimum distances: contact the utility owner to de-energize, relocate, or insulate the lines

D. Temporary Wiring and Panel Safety

Temporary wiring must be installed by a licensed electrician and meet code requirements

Panel covers must be in place at all times — exposed busbars are a direct contact and arc flash hazard

Extension cords: inspect before each use (intact jacket, no exposed conductors, ground pin intact, no splices or tape repairs). Remove damaged cords from service immediately.

Do not daisy-chain extension cords or run them through doorways where they can be pinched or damaged

E. Arc Flash Awareness

Arc flash: an electrical explosion that produces extreme heat (up to 35,000 degrees F), intense light, pressure blast, and shrapnel

Can occur when working on or near energized equipment — accidental contact, tool slip, contamination

Arc flash boundaries must be established for energized work — appropriate PPE (arc-rated clothing, face shield, gloves) required within the boundary

Best prevention: de-energize, lockout, and verify zero energy BEFORE any work on electrical systems

F. Wet Conditions

Water is an excellent conductor — wet conditions dramatically increase electrical shock risk

Never operate electrical equipment (tools, cords, panels) while standing in water or on wet surfaces without GFCI protection

Keep electrical panels, temporary power boxes, and cord connections elevated and protected from water

During rain: cover temporary power connections, suspend use of corded tools if conditions are severe

G. Emergency Response for Electrical Contact/Shock

DO NOT touch the victim if they are still in contact with the electrical source — you will be electrocuted

Disconnect the power source if you can safely reach the switch/breaker/plug

If you cannot disconnect power: use a NON-CONDUCTIVE object (dry wood, fiberglass) to separate the victim from the source

Call 911 immediately — electrical injuries often involve internal burns and cardiac arrhythmia not visible externally

Begin CPR if the victim is not breathing or has no pulse — AED if available

For equipment contact with overhead powerlines: operator stays in the cab, everyone else stays clear (ground voltage gradient). If the operator must evacuate (fire), they JUMP clear with both feet together — do not step down (step potential).

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. What does GFCI stand for and how quickly does it disconnect power during a ground fault?

Answer: _______________________________________________

Q2. How often must GFCIs be tested on construction sites?

Answer: _______________________________________________

Q3. What is the lockout/tagout procedure in the correct sequence?

Answer: _______________________________________________

Q4. What is the minimum approach distance to an overhead powerline carrying up to 750 volts?

Answer: _______________________________________________

Q5. What should you do if you find an extension cord with a missing ground pin?

Answer: _______________________________________________

Q6. What is arc flash and what temperature can it reach?

Answer: _______________________________________________

Q7. Why are wet conditions particularly dangerous around electrical equipment?

Answer: _______________________________________________

Q8. If someone is being electrocuted by a live wire, what should you do FIRST?

Answer: _______________________________________________

Q9. Why should an equipment operator stay in the cab if the equipment contacts a powerline?

Answer: _______________________________________________

Q10. Can you remove another worker's lockout lock from a breaker? Explain.

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 6: Electrical Safety on Construction Sites ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 7: Fire Prevention & Hot Work

Duration: 1.5 hours  |  Delivery: Classroom

## Learning Objectives

Explain the fire triangle and identify common construction fire risks

Describe the hot work permit procedure and fire watch requirements

Demonstrate fire extinguisher selection and PASS technique

Identify safe storage practices for propane, fuel, and combustible materials

## Content

A. Fire Triangle and Construction Fire Risks

Fire requires three elements: heat (ignition source), fuel (combustible material), oxygen (air). Remove any one element and the fire cannot sustain.

Construction sites have abundant fuel: wood framing, cardboard packaging, sawdust, insulation, solvents, adhesives, tarps, drywall paper

Common ignition sources on construction sites: welding/cutting, grinding, temporary heaters, electrical faults, smoking, spontaneous combustion (oily rags)

Construction fires are particularly dangerous because: buildings are incomplete (no fire suppression), egress routes may be limited, fire separation is not yet installed, combustible materials are concentrated

B. Hot Work Permit Procedure

Hot work = any activity producing sparks, flame, or heat: welding, cutting, brazing, grinding, soldering, torch-applied roofing

Permit required for ALL hot work outside designated shop areas

Process: hazard assessment (35-foot radius), combustible clearance, fire extinguisher placement, fire watch assignment, permit issuance by authorized person

Permit valid for ONE shift only — new permit required each day/shift

See SOP-CON-002 for detailed hot work permit procedure

C. Fire Watch Requirements

Trained person with sole responsibility: watch for fire during and after hot work

Must have fire extinguisher within reach and know how to use it

Continuous observation during all hot work operations

Post-work fire watch: MINIMUM 60 minutes after the last spark/flame. Extended to 120 minutes in high-risk environments.

Fire watch has authority to STOP hot work if unsafe conditions develop

Fire watch may NOT leave the post until relieved or the observation period is complete

D. Fire Extinguisher Types and PASS Technique

ABC dry chemical: most common on construction sites — effective on Class A (ordinary combustibles), Class B (flammable liquids), and Class C (electrical fires)

Minimum 20-lb ABC extinguisher within 10 feet of hot work locations. General coverage: extinguishers within 75 feet of all work areas.

PASS Technique: P = Pull the pin. A = Aim at the base of the fire (not the flames). S = Squeeze the handle. S = Sweep side to side at the base.

Important: a 20-lb extinguisher provides approximately 20–30 seconds of discharge. If the fire is not controlled quickly, evacuate and call 911.

Monthly visual inspection: check gauge (needle in green zone), pin in place, tamper seal intact, no visible damage, clear access

E. Combustible Material Management

Keep work areas clean — sawdust, wood shavings, cardboard, and packaging are fuel. Clean up daily.

Store flammable liquids (solvents, adhesives, fuels) in approved containers in designated storage areas — away from ignition sources, electrical panels, and exit routes

Oily rags (linseed oil, stain, varnish): dispose in self-closing metal containers. Spontaneous combustion is a real risk.

No smoking on construction sites except in designated smoking areas — enforced universally

F. Propane and Fuel Storage

Store propane cylinders upright, secured against tipping, in well-ventilated areas away from heat sources and ignition sources

Maximum storage quantities per area — per fire code and local regulations

Fuel storage (diesel, gasoline): approved containers, labeled, secondary containment, away from buildings and ignition sources

No refueling of equipment near buildings or in enclosed spaces

G. Temporary Heating Safety

Propane torpedo heaters (salamanders): maintain clearance from combustibles (minimum 10 feet), ensure adequate ventilation (CO risk), never leave unattended

Indirect-fired heaters: preferred for enclosed spaces — combustion gases are vented outside, only warm air enters the space

Hydronic heating systems: safest option for enclosed concrete curing — no combustion gases, no ignition risk

All temporary heating equipment must have automatic shut-off (tip-over switch, overheat protection)

H. Emergency Evacuation on Construction Sites

Know the alarm signal (air horn, siren, radio call) — evacuation alarm must be audible across the entire site

Proceed to the designated muster point immediately — do not stop to collect tools or personal items

Foremen conduct headcounts of their crews and report to the Superintendent

No one re-enters the site until the Superintendent gives the ALL CLEAR signal

Construction sites have unique egress challenges: incomplete stairs, missing handrails, temporary access routes — know your egress route BEFORE an emergency

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. What are the three elements of the fire triangle?

Answer: _______________________________________________

Q2. What is the minimum duration for a post-hot-work fire watch?

Answer: _______________________________________________

Q3. What does PASS stand for in fire extinguisher use?

Answer: _______________________________________________

Q4. What type of fire extinguisher is most commonly used on construction sites?

Answer: _______________________________________________

Q5. How long does a 20-lb fire extinguisher typically last during discharge?

Answer: _______________________________________________

Q6. Why are oily rags a fire hazard and how should they be disposed of?

Answer: _______________________________________________

Q7. What is the minimum clearance distance between a propane torpedo heater and combustible materials?

Answer: _______________________________________________

Q8. How often must fire extinguishers receive a visual inspection?

Answer: _______________________________________________

Q9. For how long is a hot work permit valid?

Answer: _______________________________________________

Q10. What should you do during a site evacuation — can you stop to grab your tools?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 7: Fire Prevention & Hot Work ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Session 8: Environmental Compliance on Construction Sites

Duration: 1.5 hours  |  Delivery: Classroom

## Learning Objectives

Explain erosion and sediment control requirements and common measures used on construction sites

Describe spill prevention and response procedures for fuel, oil, and chemical spills

Identify waste management requirements including proper sorting and hazardous waste handling

Recognize dust, noise, and vegetation protection responsibilities

## Content

A. Erosion and Sediment Control

Construction sites are the largest source of sediment pollution — exposed soil erodes rapidly during rain events

Regulatory requirement: prevent sediment-laden water from leaving the site and entering storm drains, waterways, or neighbouring properties

Silt fences: installed along the downslope perimeter of disturbed areas. Fabric must be trenched into the ground (6-inch minimum). Inspect after every rain event — repair or replace damaged sections.

Check dams: small dams of rock or sandbags placed in ditches and swales to slow water flow and allow sediment to settle

Stabilized construction entrances: aggregate pad at site entrance to remove mud from vehicle tires before they reach public roads

Temporary seeding and mulching: apply to disturbed areas that will remain inactive for more than 30 days

Inlet protection: filter fabric, straw wattles, or gravel berms around storm drain inlets to prevent sediment entry

B. Spill Prevention and Response

Potential spill sources: equipment fuel and hydraulic oil, concrete wash water, paint and solvents, adhesives, curing compounds

Prevention: secondary containment for fuel storage (110% of largest container volume), drip pans under equipment during maintenance, equipment inspected daily for leaks

Spill kits: located at fuel storage areas, equipment staging areas, and at minimum one per active work area. Contents: absorbent pads, booms, granular absorbent, PPE (gloves, goggles), disposal bags.

Response procedure: (1) Stop the source if safe. (2) Contain the spill — deploy absorbent booms to prevent spread. (3) Absorb — apply pads or granular absorbent. (4) Notify — report to Superintendent and Safety Officer. (5) Clean up — collect contaminated materials in labeled disposal bags. (6) Report — document the spill (type, quantity, cause, response actions, disposal method).

Reportable quantities: spills exceeding regulatory thresholds (varies by substance and jurisdiction) must be reported to the Ministry of Environment / EPA within specified timeframes

C. Waste Management

Sorting: General waste (landfill), Recyclable materials (metals, cardboard, clean wood, drywall, concrete), Hazardous waste (paint, solvents, adhesives, contaminated soil, asbestos)

Designated bins clearly labeled at convenient locations throughout the site — separate bins for each waste stream

Dumpsters must have covers to prevent rain from entering and creating leachate

Hazardous waste: stored separately in labeled, closed containers. Manifested for disposal by a licensed hazardous waste hauler. Records maintained for minimum 3 years.

Concrete washout: designated washout area lined with poly. Never wash concrete into storm drains, ditches, or on the ground.

Waste minimization: [COMPANY NAME] promotes reducing waste through accurate ordering, material reuse, and recycling. Track diversion rates per project.

D. Dust Control

Sources: excavation, grading, concrete cutting, demolition, aggregate stockpiles, unpaved roads

Water trucks: apply water to unpaved roads, stockpiles, and active earthwork areas — frequency depends on temperature, wind, and humidity

Calcium chloride: applied to unpaved roads for longer-lasting dust suppression

Tarping: cover stockpiles and open loads during transport and when not actively being used

Cutting operations: use wet cutting methods for concrete, masonry, and asphalt to suppress silica dust

Wind days: if dust cannot be controlled, suspend the dust-generating activity

E. Noise Management

Municipal noise bylaws: restrict construction hours (typically 7 AM to 7 PM weekdays, limited weekend hours — check local bylaws)

Equipment maintenance: well-maintained equipment produces less noise — mufflers, engine covers, hydraulic system maintenance

Scheduling: plan high-noise activities (pile driving, concrete breaking, heavy equipment) during permitted hours and away from sensitive receptors (hospitals, schools, residences)

Community relations: notify neighbours of anticipated high-noise activities in advance — a phone call or door hanger prevents complaints

Worker protection: hearing protection (plugs or muffs) required when noise exceeds 85 dBA — audiometric testing program per regulation

F. Tree and Vegetation Protection

Tree protection zones: established by the project arborist, typically at the drip line. No excavation, material storage, equipment operation, or grading within the tree protection zone.

Tree protection fencing: rigid fencing (not caution tape) at the drip line or as specified by the arborist

Root cutting: if roots must be cut, use clean cuts with a sharp saw — never rip or crush roots with equipment

Municipal tree bylaws: many municipalities require permits for tree removal and impose significant fines for unauthorized damage to protected trees

G. Dewatering and Discharge

Groundwater or surface water that accumulates in excavations must be managed properly before discharge

Sediment removal: pump through a settling tank, filter bag, or sediment pond before discharge to storm sewer

Discharge permits: may be required from the local conservation authority or municipal water authority — check before pumping

pH monitoring: concrete contact raises pH above acceptable discharge limits — neutralization may be required

Never pump contaminated water (fuel, oil, chemical) — contain and dispose through a licensed waste hauler

H. Environmental Incident Reporting

Report ALL environmental incidents to the Superintendent and Safety Officer immediately

Incidents include: spills (any quantity), unauthorized discharge, sediment release to waterways or storm drains, tree damage, wildlife disturbance, dust or noise complaints

Document: date, time, location, substance, estimated quantity, cause, response actions taken, photos

Regulatory notification: required for reportable quantities or incidents affecting waterways — Superintendent/Safety Officer will determine and execute the notification

Corrective actions: identify root cause, implement measures to prevent recurrence, document and file

### Knowledge Assessment Quiz

Answer the following questions. Minimum passing score: 80%.

Q1. Name two erosion control measures commonly used on construction sites.

Answer: _______________________________________________

Q2. What is the correct sequence for responding to a fuel spill on a construction site?

Answer: _______________________________________________

Q3. Into what three categories should construction waste be sorted?

Answer: _______________________________________________

Q4. Where should concrete washout water be directed — and where should it NEVER go?

Answer: _______________________________________________

Q5. Name two methods of dust control used on construction sites.

Answer: _______________________________________________

Q6. What are typical permitted construction noise hours under most municipal bylaws?

Answer: _______________________________________________

Q7. What defines a tree protection zone and what activities are prohibited within it?

Answer: _______________________________________________

Q8. Why must dewatering discharge be treated before entering the storm sewer system?

Answer: _______________________________________________

Q9. What should you do if you discover a hydraulic oil leak from equipment on site?

Answer: _______________________________________________

Q10. Who must be notified of an environmental incident on a [COMPANY NAME] construction site?

Answer: _______________________________________________

Score: _____ / 10    |    Pass / Fail    |    Evaluator Initials: _____

--- Sign-Off: Session 8: Environmental Compliance on Construction Sites ---

I confirm I have attended this training session, understood the material presented, and will apply these practices on [COMPANY NAME] construction sites.

Employee Name (Print): ________________________    Employee Signature: ________________________

Date: ________________________    Instructor: ________________________

# Training Tracker — Attendance Record

Use this tracker to record attendance for all eight training sessions. Each participant must attend and pass (80% quiz score) all sessions to receive the Certificate of Completion.

| # | Employee Name | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |

Legend: Mark attendance with date (MM/DD) and quiz score (e.g., '07/15 — 9/10'). Minimum passing score: 80%.

# Certificate of Completion

# [COMPANY NAME]

# Construction Safety Training Program

CERTIFICATE OF COMPLETION

This certifies that

_________________________________________

has successfully completed the [COMPANY NAME] Construction Safety Training Program, consisting of 8 sessions (17 hours) covering: Working at Heights & Fall Protection, Excavation & Trenching Safety, Scaffold Safety, Confined Space Awareness, Crane & Rigging Safety, Electrical Safety, Fire Prevention & Hot Work, and Environmental Compliance.

Date: ________________________

_________________________________________

Director of Health & Safety / Training Coordinator

_________________________________________

Site Superintendent / Project Manager